Cisco management devices, Console Telnet, SSH

Device Management

- Traffic that network administrator uses to configure network devices is Management.

- Management plane traffic is usually consisting protocol traffic like Telnet, SSH or SNMP.

- Management plane provides the ability to manage network infrastructure devices.

- First step toward management is to set username and password.

Console Port

- Every Cisco Router, Firewall or a Switch has a console port.

- Console port also known as the management port on its backside.

- Console port is used to connect a computer directly to a router or switch.

- It manages the router or switch since there is no display device for a router or switch.

- The console port must be used to initially to install routers.

- There is no network connection initially to connect using SSH, HTTP or HTTPS.

- Normally router, switch or firewall console port is a RJ45 port.

- Console port is the management port, which is used by administrators.

- This will allow us to work virtually on console of switch for practice.

- After just setting p/w then setting username and p/w to secure console.

Setting up Username And P/W

- Now these are in local data base so in configuration mode it must be specified login local.

Log in User Detail:

VTY (Virtual Terminal Lines):

- VTY stands for Virtual Terminal Lines or Virtual Teletype.

- VTY is a Command Line Interface (CLI) created in a router.

- VTY is used to facilitate a connection to the daemon via Telnet.

- VTY is just a way of accessing the switches or routers CLI remotely.

- Virtual Terminals are logical connections from the network to the router.

- VTY are typically telnet connections to switches or routers.

- Telnet is use to manage routers or switches remotely.

- A Telnet client and server application ships with Cisco's IOS software.

- Telnet commonly uses TCP port 23 to connect to devices.

Assigning IP to Router For Telnet(Teletype Network)

Assiging IP to Switch(Layer 2[no switch])

- SVI (Switch virtual Interface) allows to assign IP in Layer 2 Switch for Telnet

Show IP For Telnet:

TELNET:

- It allows us to remotely control switch means the router or any device connected to switch using the terminal of that device we can configure Switch.

- In this picture we used router terminal to access switch Router is also connected to switch.

Check Accessed User of Switch:

Set Username & P/W for TELNET:

SSH (Secure Shell):

- SSH stands for Secure Shell.

- SSH provides a secure remote access connection to network devices.

- SSH are two versions SSH Version I and SSH Version 2.

- Communication between the client & server is encrypted in both SSH version.

- SSH, version 2 is more secure than version SSH Version 1.

- SSH commonly uses TCP port 22 to connect to devices.

- SSH, as the preferred management protocol under the VTY interfaces.

- SSH provides a secure and reliable mean of connecting to remote devices.

- SSH, Version 2 is the more secure and commonly used version.

- SSH, require an IOS image that supports crypto features.

- SSH is a more secure way to configure routers, switches or firewalls.

- SSH requires a RSA public and private Key pair.

- Router & Switch Name should be different like sw (no). additinaly domain is required to generate key.