- Explain the Role of DHCP

- Configure and verify DHCP Client and Relay

DHCP (Dynamic Host Configuration Protocol):

- DHCP is an abbreviation for Dynamic Host Configuration Protocol.

- Cisco IOS features known as Easy IP Lease which is full-featured DHCP server.

- DHCP is a client server protocol that automatically provides an IP to hosts.

- DHCP also provide IP, subnet mask, default gateway, DNS server & other config.

- DHCP allows a network device to dynamically receive IP address parameters.

- DHCP process follows DORA process, Discover, Offer, Request & Acknowledgement.

- DHCP is app layer protocol used by hosts for obtaining network setup information.

- DHCP Server dynamically configures a host or Network device in a network.

- DHCP is Client server protocol, which uses User Datagram Protocol services.

- DHCP port number for server is UDP port 67 and for the client is UDP port 68.

- DHCP assigned Internet Protocol (IP) address from a pool (range) of addresses.

- Dynamic Host Configuration Protocol (DHCP) is an application layer protocol.

Advantages of DHCP:

- Primary advantage of DHCP is easier management of IP addresses.

- Centralized network client configuration.

- DHCP greatly reduce the time required to configure & reconfigure computers.

- DHCP Server assigning IP addresses automatically avoid configuration errors.

- Ease of adding new clients to a network.

- Reuse of IP addresses reducing the total number of IP addresses.

- No need to reconfigure each client separately.

- Configure the network from a centralized area.

- Easy handling of new users and reuse of IP address can be achieved.

IOS DHCP Server:

- Cisco devices running Cisco software include DHCP server.

- Cisco router can be used as a DHCP server and the various options available.

- Cisco IOS routers and layer 3 switches can be configured as DHCP server.

- The Cisco IOS DHCP server is a full DHCP server implementation.

- Cisco IOS DHCP assigns and manages IP addresses from specified address pools.

- Cisco IOS DHCP server can be configured to assign additional parameters as well.

- DHCP server is the one responsible for handing out IPs to the clients.

DHCP Client:

- DHCP client is a host using DHCP to obtain configuration parameters.

- The endpoint that receives configuration information from a DHCP server.

- DHCP client is anything needing an IP address that is not configured as a static.

- Cisco routers can be configure as both DHCP servers and DHCP clients.

- DHCP clients use UDP broadcasts to send their initial DHCPDISCOVER messages.

- DHCP Client uses UDP port 67 to send messages to the server.

DHCP Relay:

- DHCP relay agent is any host that forwards DHCP packets between clients and servers.

- DHCP Relay allows clients to obtain DHCP info from a server on a different subnet.

- Relay agents are used to forward requests and replies between clients and servers.

- Relay agents are used to forward request when they are not on the same physical subnet.

- The devices that do the forwarding are referred to as DHCP relay agents.

- Relay agents forward packets differently than normal IP forwarding.

- Relay agents receive a DHCP message & generate a new message out another interface.

- DHCP relay agent adds a GIADDR (Gateway Address of Packet) field.

- DHCP relay also add the Relay agent information option 82 if enabled.

DHCP Operation:

- DHCP important messages exchanged between a DHCP client & DHCP Server.

- Four basic phases are required in DHCP operations between a DHCP server and DHCP client.

- DHCP operation process for IPv4 called DORA process.

DHCP Discover Message:

- First message generated in the communication process between server and client.

- Client host generated this message to discover DHCP server in a network.

- This message is broadcasted to all devices present in a network to find DHCP server.

- In case no respond from a DHCP server, windows client assigns itself, an APIPA.

- DHCP Discover message use broadcast IP address and broadcast MAC address.

- This message is 342 or 576 bytes long & Destination MAC address is FFFFFFFFFFFF.

- Source IP address is 0.0.0.0 because DHCP client has no IP address until now.

- Destination IP address is broadcast 255.255.255.255.

DHCP Offer Message:

- DHCP servers receive a DHCP Discover message respond with a DHCP Offer message.

- DHCP Server response to client and offers the client an IPv4 address lease.

- DHCP Offer message is broadcasted by server.

- DHCP Offer size of message is 342 bytes.

- If more than, one DHCP servers then client will accept the first DHCP OFFER message.

- Server ID is specified in the packet in order to identify the server.

- The DHCP Offer message is broadcast by the DHCP server.

- The destination IP address is broadcast IP address.

- The destination MAC address FFFFFFFFFFFF

- The source IP address is server IP address & MAC address is server MAC address.

DHCP Request Message:

- Clients accept first offer received by broadcasting a DHCP Request message.

- Client receives an offer message & responds by broadcasting a DHCP request message.

- Client produce a gratuitous ARP to find any other host in network with same IP address.

- If there is no reply by other host, then there is no host with same TCP configuration.

- Broadcast message is send to server showing the acceptance of IP address.

- Source IP address is 0.0.0.0, as the client has no IP right now.

- Destination IP address is 255.255.255.255 broadcast IP address.

- Source MAC address is client MAC address & destination MAC address is FFFFFFFFFFFF.

DHCP Acknowledgment Message:

- Server accepts the client request and DHCP Acknowledgment message send.

- Server make entry with specified client ID & bind the IP address offered with lease time.

- Finally, client have the IP address provided by DHCP server.

- Server will not provide this IP address to any other host.

Configure DHCP

R1(config)#[assign ip to internal interface e0/0 192.168.1.1/24]

R1(config)#ip dhcp exclude-address 192.168.1.1 192.168.1.100 [do not assign to any client ip from this range]

R1(config)#ip dhcp pool abc

R1(dhcp-config)#network 192.168.1.0 /24 [here dhcp became R1]

R1(dhcp-config)#dns-server 8.8.8.8 8.8.2.2 2.2.2.2 [here 3 dns servers are assigned if one is down another will be used]

R1(dhcp-config)#domain-name test.com

R1(dhcp-config)#default-router 192.168.1.1

R1#show running-config | sec dhcp

Client Receiving IP:

R2(config)#inter f0/0

R2(config-if)#ip address dhcp

Configure DHCP Using Relay: