Syslog:

- Syslog stands for System Logging, standard protocol used to send system log.

- Cisco network devices Routers and Switches use Syslog to send system messages.

- Cisco network devices use debug output to a local logging process inside the device.

- Syslog is used on a variety of devices to give system information to the system admin.

- Most Cisco devices use the syslog protocol to manage system logs and system alerts.

- Logging can be used for fault notification, network forensics, and security auditing.

- Syslog messages can be output to the console, local buffer or a remote syslog serve.

- Logs can include content flow, configuration changes and new software installs etc.

- Logging helps to detect unusual network traffic, network device failures, issue etc.

- The standard port for syslog is UDP port 514. However, TCP port 6514 is also used, especially when TLS encryption is enabled for security

EXAMPLE:

| TIMESTAMP | Mar 20 17:28:10.389: Time stamp |
| --- | --- |
| FACILITY | %LINK |
| SEVERITY | 3 |
| MNEMONIC | UPDOWN |
| MESSAGE TEXT | Interface e0/2, changed state to up |

Commands:

Show logging  [it will show all the logs issued by device but limited 4096 bytes buffer]

R1#logging on-success log

R1#logging on-failure log

Local Logging:

- Everything happens on router or switch can be logged.

- By default, syslog messages are only displayed to the console.

- Because the logging console command is enabled by default.

- By default, the router sends all log messages to its console port.

- Only users physically connected to the router console port can view messages.

- This can be turned off with the no logging command.

- For local logging, Cisco IOS can save syslog messages to the internal buffer.

- Syslog messages can be output to the console or a remote syslog server.

- The logging is basically the process that generated the syslog message.

Terminal Logging

- It is like console logging, but it displays log messages to the router's VTY lines instead.

- This is not enabled by default. To enable it to use this command: R1# terminal monitor

Buffered Logging:

- This type of logging uses Cisco Router's & Switches RAM for storing log messages.

- Buffer has fixed size to ensure that the log will not deplete valuable system memory.

- Router accomplishes this by deleting old messages as new messages are added.

- To enable it use configuration mode command: R1 (config)# logging buffered

Syslog Server Logging:

- Router can use syslog to forward log messages to external syslog servers for storage.

- Syslog Server Logging method of type of logging is not enabled by default in devices.

- R1 (config)# logging #serverIP

3CDaemon is application used as a admin for monitoring purpose. Multiple routers syslog messages can be shown into one place like admin can view what and where changes are being configured.