Network Topology Architecture  
2-Tier & 3-Tier Architectures

# Three Tier:

- The Three Tier (3 Tier) Architecture is a great solution for large enterprises.

- Cisco suggests 3 Tier hierarchical network model, that consists of three layers.

- The Three Tier are the Core layer, the Distribution layer, and the Access layer.

- Cisco Three-Layer network model is the preferred approach to network design.

- Maximizes performance, network availability & ability to scale network design.

Core Layer:

- Core Layer consists of biggest, fastest & most expensive routers with highest model.

- In the Three Tier Architecture, the Core Layer is the one coordinating everything.

- Its only role is to forward traffic, the fastest it can, here you don't apply any policy.

- Where several distribution switches, Core Layer is considered as backbone of networks.

- Core Layer routers or switches are used to merge geographically separated networks.

- The Core Layer routers or switches move information on the network as fast as possible.

- Core switches talk with distribution switches using dynamic routing protocols, like OSPF.

- At this layer, most advanced & expensive switches used ones with modular form factor.

- The Core Layer can provide the high-speed switching, reliability and fault tolerance.

Distribution Layer:

- The Distribution Layer is located between the Access Layer and the Core Layers.

- This layer is to provide boundary definition by implementing access lists & other filters.

- The Distribution Layer defines policy for network and include high-end Layer 3 switches.

- Distribution Layer ensures that packets are properly routed between subnets and VLANs.

- The Distribution Layer can provide, Aggregation of Local Area Network or WAN links.

- This Layer provide Policy-based security in form of access control lists (ACLs) and filtering.

- Also Provide, Routing services between LANs and VLANs and between routing domains.

- Distribution Layer provide Redundancy, load balancing, aggregation & summarization.

- Broadcast domain control, routers or multilayer switches do not forward broadcasts.

Access Layer:

- The Access Layer is the one closer to the users, at this layer we find the users themselves.

- The Access layer includes access switches which are connected to the end devices.

- Access layer switches ensures that packets are delivered to the end points or devices.

- The main purpose of the Access Layer is to physically connect users to the network.

- There is just a cable between end-user or end points PCs and access-layer switches.

- These are security policies we want to enforce in order to allow access to the network.

- For example, we can configure port-security and Network Access Control in this layer.

- The Access Layer can provide, Layer 2 switching, High availability, and Port security etc.

- Also provide, QoS classification, address Resolution Protocol (ARP) inspection & VLANs.

- Virtual access control lists (VACLs), Spanning tree, Power over Ethernet (POE) and VoIP.

# Two Tier:

- Two Tier design model is more suitable for small to medium-size campus networks,

- Core & distribution functions can be combined into one, also known as collapsed core.

- Collapsed Core is when distribution & core layer functions are implement by single device.

- The primary motivation for the collapsed core design is to reduce the network cost.

- In two Tier still maintaining most of the benefits of the three-tier hierarchical model.

## Benefits of Hierarchical Model:

- Main benefits help to design, deploy & maintain scalable hierarchical internetwork.

- Three-Layer or 2-layer network model allows in creating high performance networks.

- It allows better network management and isolate causes of network trouble & issue.

- Cisco Three Layer Network Model allows better filter & policy creation application.

- Cisco Three Layer Network Model allows us to efficiently accommodate future growth.

- Three Layer or Two Layer Network Model provides better redundancy & availability.

- Multiple links across multiple devices provides better redundancy and availabilities.

- If one switch is down, we have another alternate path to reach the destination.

- When the network grows, we can easily add more distribution or access layer switches.