sConfigure and verify IPv4 Address

Configure and verify IPv4 Addressing

- IP stand or Internet Protocols internet means international Network.

- Protocols are set of rules and regulation.

- V stand for version, where VI, V2, V3, V5 and V6.

- VI, V2, and V3 were all test versions that were improved.

- IPv5 also called the internet steam protocol developed in 1980s.

- IP address is a logical address for a Network Adapter.

- The IP address uniquely identifies computers on a TCP/IP Network.

- IP address is a numeric identifier that is assigned to a device for communication.

- The designers of the Internet Protocol defined an IP address as a 32-bit number.

- IPv4 address consists of 32 bits, which limits the address space to 4294967296 (232).

- IP address consists of four decimal numbers, which separated by dots or decimal points.

- Hence, we also called IP Address version 4 dotted-decimal notation.

- For example, 192.168.1.0 is an IP address written in dotted-decimal notation.

- The binary version of 192.168.1.0 is 11000000 10101000 00000001 00000000.

- Each decimal numbers of IP address represents 8-bit, and called an octet.

- Hence, an IP address represents 32-bit (or 4 bytes).

- The range of each octet is between O and 255.

- IPV4 address start rom 0.0.0.0 and end with 255.255.255.255.

- It is also called 32 bit, logical, network, variable, software, layer 3 address.

IPv4 address space can be subdivided into five classes.

IPV4 Classes

- IPv4 address space can be subdivided into five classes

- IPv4 class are class A, class B, class C, class D, and class E.

- IPv4 each class consists of subset of the overall IPv4 address range.

- IPv4 class A, class B and class C used for Unicast ranges.

- IPv4 Class D used for Multicast Range.

- IPV4 Class E reserved for Research purpose.

- IPv4 Class E network is reserved for "experimental use".

- IPv4 Class E should not be assigned to host devices.

- IPv4 address range from 0.0.0.0 through 0.255.255.255 cannot work.

Loopback Address:

- Address beginning with 127 is unacceptable to assign them any network host.

- From 127.0.0.0 to 127.255.255.255 is fully reserved for loopback purpose.

- Loopback interface allows IT professionals to test IP software broken or corrupted.

- Loopback addresses are used for checking and troubleshooting purposes.

Unicast Address Type

- One to one communication is called Unicast Communication.

- Type of communication where data is sent from one computer to another computer.

- Unicast is a one-to-one type of network communication.

- Different data streams are generated for each Unicast connection.

- In Unicast type of communication, there is only one sender, and only one receiver.

Multicast Address Type

- One to a specific group communication is called Multicast Communication.

- Type of communication where multicast traffic addressed for a group of devices.

- IP multicast traffic are sent to a group and only members of that group receive.

- Devices, interested in a particular Multicast traffic, must join to that Multicast group.

- IP Multicast Groups are identified by Multicast IP Addresses Class D Addresses.

- In Multicast, the sender transmits only one copy of data and delivered many devices.

- Address range from 224.0.0.0 to 255.255.255.255 cannot assignment to network hosts.

- IP version 4-cIass D network is reserved for multicasting.

Broadcast Address Type

- one to all communication is ca e Broadcast Communication.

- Type of communication where data is sent from one computer and copy to all devices.

- In Broadcast, there is only one sender and the data is sent only once.

- However, the Broadcast data is delivered to all connected devices.

- Switches forward the broadcast traffic and Routers drop the broadcast traffic.

Subnet Mask:

- Class A's default mask is 255.0.0.0, or /8

- Class B's default mask is 255.255.0.0, or / 16

- Class C's default mask is 255.255.255.0, or /24

- 255.0.0.0 in binary is 11111111 00000000 00000000 00000000.

- 255.255.0.0 in binary is 11111111 11111111 00000000 00000000.

- 255.255.255.0 in binary is 11111111 11111111 11111111 00000000.

- Class A = /8 =224 Addresses for hosts

- Class B = / 16 = 216 (65,000) Addresses for hosts

- Class C= /24 = 28 (256) Addresses for hosts

HOW to find Subnet Mask:

NID: Network ID

HID: Host ID

BID: Broadcast ID

- Numerically 256 (2^8) devices can be connected with each other using class C IP address that range include [192,223] but actually only 254 devices connect because 192.1.1.0 is the network id and 192.1.1.255 is broadcast id so these ips excluded.

Network Mask:

- Network mask or NetMask is a 32-bit binary number.

- Network mask is usually written in dotted-decimal format.

- Network mask defines the size of the host part of an IP address,

- Representing the host part of the IP address with binary os in the mask.

Network Address

- Network address or network number is a number that uses dotted -decimal notation.

- The number it represents all hosts in a single Class A. B. or C IP network.

- For example, an IP address 132.168.0.1 with network\ mask 255.255.255.0.

- the give IP addre6s. the network address will be 192.168.0 0.

- Network\; address, use logical AND operation between IP address network mask

- For example, an IP address 192.168.0.1 with network mask« 255.255.255.0,

- Convert the IP address 192.168.0.1 to binary format.

- We get 11000000 10101000 00000000 0000000L

- Logical AND operation we get 11000000 10101000 00000000 00000000.

Private Addresses

- Private IP addresses are used inside the LAN for Private Communication.

- Private IP Addresses are also called Non Routable IP addresses.

- Specific address ranges within Class A, Class B, & Class C reserved for private networks.

- A private IP address is a non-internet facing IP address on an internal network.

- Large companies use Class A as it allows more than 16 million hosts.

- Class B manages 16,384 hosts per network where Class C is used 254 hosts.

APIPA

- APIPA stand for Automatic Private IP Addressing.

- Windows computer assign itself an IP address if a DHCP is not available.

- For example, this could occur on a network without a DHCP server.

- On a network, if a DHCP server is temporarily down for maintenance.

- With this feature, a Windows computer can assign itself IP address.

- If a DHCP server is not available or does not exist on the network.

- This feature makes configuring and supporting LAN running TCP/IP less difficult.

Class Full IP Address:

- Those IPv4 addresses which use default subnet mask.

Classes IP Address

- Those IPv4 addresses, which does not use default subnet mask.

- Those IPV4 addresses which subnetting are done.