Configure and verify VLAN (virtual local area network)

VLAN (Virtual Local Area Network):
Dividing a network in sub-network due broadcast issue in switch is called Vlan.

VLAN

- VLAN stands for Virtual Local Area Network.

- VLAN is a logical grouping of networking devices.

- Break large broadcast domain in smaller broadcast domains.

- VLANs divide the switch into different LAN segments.

- Consider VLAN as a subnet.

- Two different subnets cannot communicate with each other without router.

- Two different VLANs also requires router to communicate.

- VLAN used in Switches and it operates at Layer 2,

- By default, in every switch there is five VLANs.

- VLAN 1 is the default VLAN for every switch port.

- The standard VAN range is from VLAN 1 to 1005.

- The Extended VLAN range is from VLAN 1006 — 4094.

- Voice VLAN enables the access port to carry IP voice traffic.

- By default, in every cisco Switch the voice VLAN is disabled.

- VLANs are stored in Cisco Switch vlan.dat file in Flash Memory.

VLAN Analogy

- Partition hard disks on a computer into several logical partitions.

- The main purpose of hard disks partition is to organize the disk space.

- It ensures that corruption of partition does not affect another partition.

- Like have a big room separated by partitions to make them two houses for use.

- It is as big office separated by partitions makes in many offices.

- It is like a big classroom, divide into many section such as Class A. B, C etc.

- A single network is divided into 2 local networks it happened through Vlan.

- A single switch handles two lan of same network.

Types of VLAN:

Access Port

- Access port transports traffic to and from only the specified VLAN allotted to it.

- Access port will only have a single VLAN set up on the interface.

- Access port carries traffic for just a single VLAN.

- If VLAN is not configured, the interface can carry traffic for default VLAN that is VLANI.

- Accessport is port that can be assign to a single vlan.

VLAN Database

- The VLAN database IS used to store vlan data, such as eV NID, name an

- The default location of the VLAN database is in the local vlan.dat file.

- VLAN database is stored in non-volatile memory, which is flash.

Normal VLAN

- Normal VLANs range are from VLANs 1 to VLAN 1005.

- Normal range VLANs can be configured in both database configurations mode & global.

- Normal VLANs are stored in vlan.dat file in Flash memory.

- VTP versions 1 and 2 can advertise normal range VLANs only.

Extended VLANs:

- Extended VLANs are VLANs that fall in the

- range 1006 to 4094

- Extended VLAN are mainly used in se e provider networks.

- Extended VLAN allow for the provision of large numbers of customers.

- Extended VLANs differ from normal VLANs because they have higher numbers.

- Extended VLANs must be configured in VTP transparent mode or VTP V3.

- Extended VLANs are saved to the running-config.

Trunk Port

- A Trunk port is a port that is assigned to carry traffic for all the VLANs.

- In order to pass the traffic from VLANs on one switch to another switch.

- We need to implement what is called a 'trunk' port or trunk interface.

- That are accessible by a specific switch, a process known as trunking.

- It mark frames with unique identifying tags either 802.1Q/dot.1Q tags or ISL (By cisco no longer used) tags.

- Trunk Port or interface mark the traffic When move between cisco switches.

- Therefore, every single frame can be directed to its designated VLAN.

- Trunk ports are designed for interconnecting and allow one or more VLANs.

- Trunk ports are designed for interconnecting switches and support all VLANs.

- Trunk switch ports belongs to and carry the traffic of more than one VLAN.

- Each port on a switch can be configured as either an access or a trunk port.

- Trunk port is not included in any vlan.

Dot1Q/IEEE 802.1Q:

- DotlQ is IEEE 802.1Q, the standard for trunking encapsulation.

- Common trunking protocol, standard and supported by many vendors.

- On Cisco Switches, you configure IEEE 802.1Q (Dot1Q) on trunk ports.

- DotlQ allow tagged frames to be transported on a trunk link or ports.

- DotlQ allowing multiple VLANs to traverse through one link or ports.

- IEEE 802.1Q (DotlQ) extends the VLANs across the whole network.

- IEEE 802.1Q (DotlQ ) trunks use VLAN I as the default native VLAN.

- VLAN tagging is usually done based on the standard IEEE 802. IQ

- Tag includes info related to particular VLAN which frame belongs to.

- Its indicate VLAN membership within a frame going across the network.

- 802.1Q also adds a 4-byte tag into the Ethernet frame for VLAN tagging.

- IEEE 802.1Q (DotlQ) Supports both Normal and Extended range VLANs.

- IEEE 802.1Q (DotlQ) Inserts tag into existing Switches Ethernet header.

- Switch one tagged frame with hexadecimal equivalent to vlan number. Like a shows vlan 10.

DTP (Dynamic Trunking Protocol):

- DTP Cisco proprietary protocol which stand for Dynamic Trunking Protocol.

- DTP is point-to-protocol negotiates common trunking mode between Switches.

- DTP is used to dynamically build trunk links between the two Cisco Switches.

- Which is used to automatically negotiate trunks between the Cisco Switches.

- DTP is used to negotiate & form trunk connection between switches dynamically.

- DTP can also be used for negotiating encapsulation type of either IEEE 802.1Q or ISL.

- Dynamic Trunking Protocol is Layer 2 (Data Link) protocol and is enabled by default.

- The default mode can be "Dynamic Auto" or "Dynamic Desirable" depend on Switches.

- A Non-Cisco Switches does not support Dynamic Trunking Protocol (DTP) only Cisco.

- Port configures as no negotiate prevents generating Dynamic Trunking Protocol frames.

- DTP has two default timers that cannot be changed one is Hello & other is the timeout.

Native Vlan:

- Without tag, Switch will not know what VLAN the traffic belongs to.

- Switch associates untagged traffic with configured as the Native VLAN.

- A VLAN that any received untagged traffic gets assigned to on Trunk port.

- A VLAN that travel without tag it assigned to an 802.1Q trunk port.

- By default, the Native Virtu4i Local Area Network (VLAN) is one 1.

- Packet without tagged on a dotlq link belongs to Native VLAN.

- Best practice and security to change Native VLAN on all switches.

- Native VLAN ID must match on both end of the Cisco Switch trunk.

- The VLAN dotlq tag native command will tag VLAN on all trunks.

- SW(config-if) # switchport trunk native vlan vlan-id

Configure VLAN:

sw#enable

sw#config

sw(config)#vlan 10

sw(config)#name IT

sw(config)#exit

sw(config)#do show vlan br     // show vlan brief table

.....................

......................

......................

sw(config)#int f0/1

sw(config-if)#switchport access/voice vlan 10

sw(config)#exit

// similarly we can add our more devices for separate vlan